Simulation task

1 .Simulation of edge computing delay under local processing, cloud processing, latency, bandwidth, and task size

clc; clear; close all;

% Task size in MB

TaskSize = 1:10:100;

% System parameters

CPU_Edge = 2e9;       % Edge CPU frequency (cycles/sec)

CPU_Cloud = 10e9;     % Cloud CPU frequency (cycles/sec)

CyclesPerBit = 1000;  % CPU cycles required per bit

Bandwidth = 10e6;     % Bandwidth = 10 Mbps

Latency = 50e-3;      % Network latency = 50 ms

% Convert task size from MB to bits

TaskBits = TaskSize * 8e6;

% Processing delay

EdgeProcessingDelay = (TaskBits * CyclesPerBit) / CPU_Edge;

CloudProcessingDelay = (TaskBits * CyclesPerBit) / CPU_Cloud;

% Transmission delay

TransmissionDelay = TaskBits / Bandwidth;

% Total delay for local processing

LocalDelay = EdgeProcessingDelay;

% Total delay for cloud processing

CloudDelay = Latency + TransmissionDelay + CloudProcessingDelay;

% Display results

disp('Task Size (MB)    Local Delay (sec)    Cloud Delay (sec)');

for i = 1:length(TaskSize)     fprintf('%10d %20.4f %20.4f\n', ...         TaskSize(i), LocalDelay(i), CloudDelay(i)); end

% Plot delay comparison figure;

plot(TaskSize, LocalDelay, '-o', 'LineWidth', 2); hold on;

plot(TaskSize, CloudDelay, '-s', 'LineWidth', 2);

grid on;

xlabel('Task Size (MB)'); ylabel('Total Delay (seconds)'); title('Edge Computing Delay: Local Processing vs Cloud Processing');

legend('Local Processing', 'Cloud Processing');

output

Task Size (MB)    Local Delay (sec)    Cloud Delay (sec)

1               4.0000               1.6500

11              44.0000              17.6500

21              84.0000              33.6500

31             124.0000              49.6500

41             164.0000              65.6500

51             204.0000              81.6500

61             244.0000              97.6500

71             284.0000             113.6500

81             324.0000             129.6500

91             364.0000             145.6500

2 .Simulation of carrier aggregation performance in LTE-Advanced under single carrier, dual carrier, triple carrier, varying bandwidth, and SNR

clc; clear; close all;

% SNR range in dB

SNR = 0:5:30;

% Bandwidth of each carrier in MHz

BW = 20;

% Number of carriers

N1 = 1;     % Single Carrier

N2 = 2;     % Dual Carrier

N3 = 3;     % Triple Carrier

% Calculate Spectral Efficiency using Shannon Capacity

SE = log2(1 + 10.^(SNR/10));

% Throughput calculation (Mbps)

Throughput_Single = N1 * BW * SE;

Throughput_Dual   = N2 * BW * SE;

Throughput_Triple = N3 * BW * SE;

% Display results

disp('SNR(dB)   Single Carrier(Mbps)   Dual Carrier(Mbps)   Triple Carrier(Mbps)');

for i = 1:length(SNR)     fprintf('%5d %20.2f %20.2f %20.2f\n', ...

SNR(i), ...

Throughput_Single(i), ...

Throughput_Dual(i), ...         Throughput_Triple(i)); end

% Plot Carrier Aggregation Performance figure;

plot(SNR, Throughput_Single, '-o', 'LineWidth', 2); hold on;

plot(SNR, Throughput_Dual, '-s', 'LineWidth', 2); plot(SNR, Throughput_Triple, '-^', 'LineWidth', 2);

grid on;

xlabel('SNR (dB)'); ylabel('Throughput (Mbps)');

title('LTE-Advanced Carrier Aggregation Performance');

legend('Single Carrier', 'Dual Carrier', 'Triple Carrier', ...

'Location', 'northwest');

SNR(dB)   Single Carrier(Mbps)   Dual Carrier(Mbps)   Triple Carrier(Mbps)

0                20.00                40.00                60.00

5                41.15                82.29               123.44

10                69.19               138.38               207.57

15               100.56               201.11               301.67

20               133.16               266.33               399.49

25               166.19               332.38               498.56

30               199.34               398.69               598.03